THE UNIVERSITY OF DANANG

UNIVERSITY OF SCIENCE AND TECHNOLOGY

Faculty of Advanced Science and Technology

LABORATORY REPORT

INTRODUCTION TO VERY LARGE SCALE INTERGRATION IC DESIGN

Instructor		: Nguyen Van Cuong

Class		: 21ECE

Student		: Nguyen Thi Tam

Da Nang, October 18th 2024

Contents

I.	Technical Design	2

1.	Technical Objectives:	2

2.	Performance Requirements:	2

3.	Design Technique:	2

II.	Physical Design:	6

1.	Design of the Transistor for a 2-input XOR Gate:	6

2.	2-input XOR Gate:	6

3.	Combining 2-input XOR Gates:	6

III.	Logic Design:	6

1.	Logic Equation for a 4-input XOR Gate:	6

2.	Logic circuit simulation:	7

IV.	Layout Design:	8

1.	Layout Creation Process:	8

V.	Conclusion	10

# Technical Design

## Technical Objectives:

- Design a 4-input XOR gate using CMOS technology with a 0.25-micron process.

- Ensure that the circuit has a propagation delay of less than 0.5 ns.

- The circuit area must not exceed 500 square microns.

- The supply voltage is 2.5V, and the circuit must drive an external load of 20 fF.

## Performance Requirements:

- Check the propagation delay and optimize the transistor sizes to meet the required delay.

- Use simulation tools to measure the circuit's parameters.

## Design Technique:

VDD =2.5 (V) so

=115*10-630*10-6*0.6310*2.5-0.43-0.6322.5-0.4-1.02=2.7

Cint=Cgd1+Cgd2+Cdb1+Cdb1

=2CGD0nWn+2CGD0nWn+Cdb1+Cdb1

=2*0.31fFum*Wnum+2*0.27fFum*Wpum+0.66 (fF)+1.5 (fF)

=0.62 Wn(fF)+0.54*WpfF+2.16(fF)

From the requirement with external load = 20 fF, so:

CL=Cint+ Cext=0.62 WnfF+0.54*WpfF+2.16fF+20fF

= 0.62 WnfF+0.54*WpfF+22.16(fF)

For a supply voltage of 2.5 V, the normalized on-resistances of NMOS and PMOS transistors equal 13 kW and 31 kW, respectively.  (table 3.3)

- Reqn = 13kΩ; Reqp =31kΩ

tp=0.69CLReqn+Reqp2=0.69 0.62 WnfF+0.54*WpfF+22.16fFReqn+Reqp2 <0.5ns

- We must choose Wn,Wp to obtain tp < 0.5 ns

- 0.69 0.62 WnfF+0.54*WpfF+22.16fFReqn+Reqp2=0.5ns

0.62 WnfF+0.54*WpfF=0.5nsReqn+Reqp20.69- 22.16*10-15

=0.5*10-913*103+31*10320.69- 22.16*10-15=10.8(fF)

- 0.62 WnfF+0.54*WpfF=10.8(fF)

And Wp=2.7Wn

- To have propagate delay < 5ns, the size of invertor must be:

Wp <14.03 um and Wn < 5.20 um

- Choose:  Wn=0.93 um and Wp=2.50 um

So, Cint of of reference invertor  is: 0.62 WnfF+0.54*WpfF+2.16fF

=0.62 WnfF+0.54*WpfF+2.16fF=0.62*0.93+0.54*2.5+2.16=4.09 (fF)

# Physical Design:

## Design of the Transistor for a 2-input XOR Gate:

- A 2-input XOR gate can be constructed using 12 transistors (6 PMOS and 6 NMOS).

- Design the pull-up network with PMOS and the pull-down network with NMOS transistors.

## 2-input XOR Gate:

- PMOS in the pull-up network is activated when the input is low, pulling the output high.

- NMOS in the pull-down network is activated when the input is high, pulling the output low.

## Combining 2-input XOR Gates:

- To create a 4-input XOR gate, combine three 2-input XOR gates. Use a 3-stage XOR structure to calculate the output of the 4-input XOR:

  - Stage 1: Z1=A⊕B, Z2=C⊕D

  - Stage 2: Y=Z1⊕Z2 ​

# Logic Design:

## Logic Equation for a 4-input XOR Gate:

Y=A ⊕ B ⊕ C ⊕ D

Or Y = (A⊕B) ⊕ (C⊕D)

- Logic diagram:

- Truth table:

| Input |  |  |  | Output |
| --- | --- | --- | --- | --- |
| A | B | C | D | Y |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 |

## Logic circuit simulation:

- The logic signal is simulated by using tool DSCH .

# Layout Design:

## Layout Creation Process:

- Physical layout of the circuit by using tool Microwind.

- Check DRC (dessign rule check).

# Conclusion

Achieved Results: Summarize the requirements and the outcomes of the design, such as the circuit area, propagation delay, and A×tp2A \times t_p^2A×tp2​ value.

Comments: Provide comments on the achieved results, potential improvements, and practical applications of this 4-input XOR gate.