THE UNIVERSITY OF DANANG

UNIVERSITY OF SCIENCE AND TECHNOLOGY

Faculty of Advanced Science and Technology

LABORATORY REPORT

INTRODUCTION TO VERY LARGE SCALE INTERGRATION IC DESIGN

Instructor		: Nguyen Van Cuong

Class		: 21ECE

Student		: Tran Hoang Minh

Da Nang, September 3rd 2024

Index

I.	Overview of Microwind and DSCH:	3

II.	Technical design	4

1.	Requirement Specification:	4

III.	Logic design	5

1.	Top-level Interfaces:	7

2.	Verilog:	7

IV.	Circuit design	8

1.	Schematic	8

2.	Waveform	9

V.	Physical design	10

1.	Design Reference Invertor	10

2.	Layout	14

VI.	Result	16

VII.	Reference	17

- Overview of Microwind and DSCH:

Microwind is a tool for designing and simulating circuits at layout level. The tool features full editing facilities (copy, cut, past, duplicate, move), various views (MOS characteristics, 2D cross section, 3D process viewer), and an analog simulator.

DSCH is software for logic design. Based on primitives, a hierarchical circuit can be built and simulated. It also includes delay and power consumption evaluation.

Silicon is for 3D display of the atomic structure of silicon, with emphasis on the silicon lattice, the dopants, and the silicon dioxide.

The Microwind software works is based on a lambda grid, not on a micro grid. Consequently, the same layout may be simulated in any CMOS technology. The value of lambda is half the minimum polysilicon gate length. Table A-xxx gives the correspondence between lambda and micron for all CMOS technologies available in the companion CD-ROM.

- Technical design

- Requirement Specification:

| Name | 4-input XOR gate |
| --- | --- |
| Process | 0.25-micron |
| Supply voltage | 2.5V |
| Propagation delay | < 0.5 ns |
| Area | < 500 square microns |
| External load | 20 fF |

- Logic design

The XOR (Exclusive OR) gate is a digital logic gate that outputs true or high only when the two binary bit inputs to it are unequal. In other words, it outputs a 1 when the number of 1’s inputs is odd, making it essential for arithmetic functions in computers and other digital systems.

The 6-transistor XOR gate design is a very compact solution for implementing the XOR function. The
schematic diagram is shown in Figure 1. The main drawback is the use of pass transistors which may create non-ideal logic levels due to threshold voltage degradation. In short, n-channel transistors cannot transfer the level 1 correctly, the p-channel transistors cannot transfer the level 0 correctly. Other values are correctly executed. Using DSCH, the observed simulation is always correct, as DSCH do not take into account “weak-1” and “weak-0” levels

The compilation of the XOR gate using Microwind leads to a compact layout, but the non-ideal internal
levels do not induce a correct inverter simulation, which creates important negative effects:
 Very large delays
 Important static consumption

OUT = A ⊕ B ⊕ C ⊕ D = (A ⊕ B) ⊕ (C ⊕ D)

- Top-level Interfaces:

| Inputs | Outputs |
| --- | --- |
| A | output |
| B |  |
| C |  |
| D |  |

- Verilog:

// DSCH3

// 10/20/2024 4:17:22 PM

// D:\download\VLSI1\DSCH1\DSCH\examples\xor4_lan3.sch

module xor4( B,A,C,D,output);

input B,A,C,D;

output output;

wire w5,w6,;

xor #(16) xor2_1(w5,C,D);

xor #(16) xor2_2(output,w6,w5);

xor #(16) xor2_3(w6,A,B);

endmodule

// Simulation parameters in Verilog Format

always

#2000 B=~B;

#1000 A=~A;

#4000 C=~C;

#8000 D=~D;

// Simulation parameters

// B CLK 20.000 20.000

// A CLK 10.00 10.00

// C CLK 40.000 40.000

// D CLK 80.000 80.000

- Circuit design

- Schematic

- Waveform

- Physical design

- Design Reference Invertor

So we have  WLpWLn=115×10-6-30×10-6×0.63-1×2.50-0.43-0.6322.50-0.4-1.02=3.5≈3

Cint=Cgd1+Cgd2+Cdb1+Cdb1

Cint=2CGD0nWn+2CGD0nWn+Cdb1+Cdb1

Cint=2×0.31×Wn+2×0.27×Wp+0.66+1.5

Cint=0.62 ×Wn+0.54×Wp+2.16

We have  CL=Cint+ Cext

From the requirement with external load is equal 20 fF, so

CL=Cint+ Cext=( 0.62× Wn+0.54×Wp+2.16 )+20

And with he generic 0.25 um CMOS process, we have the Req for NMOS and PMOS at VDD=2.5 V is:

Reqn=13 kΩ and Reqp=31 kΩ

And with the requirement for propagation delay is  < 0.5ns.

tp=0.69CLReqn+Reqp2<0.5ns

tp=0.69[0.62 Wn+0.54Wp+2.16+20]Reqn+Reqp2<0.5ns

=>0.690.62× Wn+0.54×Wp+2.16+2013 kΩ+31 kΩ2<0.5ns

=>0.62× WnfF+0.54×WpfF<10.8 fF

And we have WLpWLn≈3 or   Wp=3 Wn

=>Wp< 17.277 μm and  Wn< 5.750 μm and Wp=3 Wn

Choose  Wp=2.500 μm and  Wn=0.750 μm

So, Cint of of reference invertor  is: 0.62 Wn+0.54Wp+2.16

=0.62 Wn+0.54*Wp+2.16=0.62*0.75+0.54*2.5+2.16=3.975 ≈4

Design the 4input  XOR gate :

The method of Logical Effort is applied with the following steps:

1. F = CL/Cint = 24/4 = 6

2. G = 4 × 4  = 16

3. B = 1

4. H = GBF = 96, so the optimal stage effort is f^ =sqrt(H) = 9.79

y = 9.79* 9.79/ 4 * 4 = 9.79

x = 9.79/2/4 * 4 = 4.89

5. So the gate sizes are

(WL)n,x=1(4.894)≈1 , (WL)p,x=3(4.894)≈3 ,

(WL)n,y=1(9.794)≈4 , (WL)p,y=3(9.794)≈12

The XOR2 gate delay is 	d1 = g1h1 + p1 = 4/2 × 1 + 4 = 6

d2 = g2h2 + p2 = 4 × 1 + 4 = 8

Hence, the path delay is 14. Recall that delay is expressed in units of τ. In a 0.25-micron process with τ = 61ps, the delay is 854ps

- Layout

- Result

Functionality: the waveform observed demonstrates that physical design is functioning correctly as 4-input xor gate.

Speed: Confirm that the propagation delay meets the 0.5 ns specification.

Area: Validate that the area is within acceptable limits (24.8 x 13.3 = 330 square microns)

- Reference

Microsoft Word - Book appendixA e.doc

jntuh vlsi lab - Research is Fun

Microsoft PowerPoint - MohantyVLSI5Microwind

https://www.bing.com/ck/a?!&&p=6feccc956365be4c68b9e00ccda95e9aa40e17470bb1d078f7df26e572b0200eJmltdHM9MTczMDUwNTYwMA&ptn=3&ver=2&hsh=4&fclid=3bdf4c52-f7fb-6750-3bff-4348f69d6642&psq=xor+in+microwind&u=a1aHR0cHM6Ly9taWNyb3dpbmQubmV0L2Rvd25sb2FkLzE3&ntb=1